Supplemental Information: Gas Chromatography Analysis and Data

Analysis of the residual oil remaining on the washed and rinsed feathers (i.e., the extract) was carried out on an HP 5890 gas chromatograph with an autosampler, using a 25 m x 0.2 mm x 0.33 μm nonpolar HP Ultra 1 dimethylpolysiloxane column. GC conditions are listed below.

Supplemental Table 1: GC conditions

| Condition | Setting |
| --- | --- |
| Injector Temperature | 285°C |
| Detector Temperature | 250°C |
| Head pressure | 20 psi; 40 mL/min on split vent |
| Oven program | 80°C for 1 minute then 10°C/min to 190°C, then 25°C/min to 300°C, hold 5.6 minutes |

Supplemental Table 2: Instrument response factors relative to 1-octadecene = 1.00 (calculated as mass ratio of component to standard divided by the area ratio of component to standard)

| o-xylene | 0.6453 |
| --- | --- |
| mesitylene | 0.6526 |
| n-butylcyclohexane | 0.6512 |
| n-butylbenzene | 0.6040 |
| tetralin | 0.5894 |
| naphthalene | 0.5684 |
| n-dodecane | 0.6063 |
| tetradecane | 0.6133 |
| n-eicosane | 1.6996 |

Supplemental Table 3:  GC Data

| Day 1 | Trial 1 |  | Trial 2 |  | Trial 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | retention time (min) | area count | retention time (min) | area count | retention time (min) | area count |
| Control | mass of oiled feathers (g) | 2.03 | mass of oiled feathers (g) | 2.01 | mass of oiled feathers (g) | 2.01 |
| o-xylene | 6.475 | 2.24E+05 | 6.466 | 2.53E+05 | 6.502 | 1.96E+05 |
| mesitylene | 7.604 | 2.99E+05 | 7.595 | 3.49E+05 | 7.632 | 2.72E+05 |
| n-butylcyclohexane | 8.807 | 2.24E+05 | 8.797 | 2.57E+05 | 8.837 | 2.08E+05 |
| n-butylbenzene | 9.023 | 1.88E+05 | 9.015 | 2.23E+05 | 9.054 | 1.76E+05 |
| tetralin | 10.835 | 3.39E+05 | 10.828 | 4.02E+05 | 10.869 | 3.19E+05 |
| naphthalene | 11.193 | 3.42E+05 | 11.19 | 4.09E+05 | 11.23 | 3.21E+05 |
| n-dodecane | 11.447 | 3.07E+05 | 11.445 | 3.62E+05 | 11.484 | 2.93E+05 |
| tetradecane | 14.303 | 3.36E+05 | 14.305 | 3.91E+05 | 14.344 | 3.18E+05 |
| 1-octadecene | 18.425 | 3.56E+04 | 18.428 | 4.62E+04 | 18.491 | 3.67E+04 |
| n-eicosane | 19.924 | 6.92E+04 | 19.927 | 9.81E+04 | 19.994 | 8.00E+04 |
| Dawn Ultra Pure Essentials | Mass of oiled feathers (g) | 2.02 | Mass of oiled feathers (g) | 2.02 | Mass of oiled feathers (g) | 2.03 |
| o-xylene | 6.448 | 1.13E+04 | 6.456 | 1.16E+04 | 6.505 | 1.84E+04 |
| mesitylene | 7.565 | 1.52E+04 | 7.573 | 1.81E+04 | 7.626 | 2.81E+04 |
| n-butylcyclohexane | 8.769 | 1.10E+04 | 8.779 | 1.28E+04 | 8.833 | 2.10E+04 |
| n-butylbenzene | 8.986 | 9.80E+03 | 8.996 | 1.25E+04 | 9.052 | 1.92E+04 |
| tetralin | 10.781 | 1.80E+04 | 10.795 | 2.43E+04 | 10.853 | 3.71E+04 |
| naphthalene | 11.136 | 1.87E+04 | 11.15 | 2.59E+04 | 11.209 | 3.90E+04 |
| n-dodecane | 11.398 | 1.57E+04 | 11.412 | 2.07E+04 | 11.469 | 3.41E+04 |
| tetradecane | 14.26 | 1.75E+04 | 14.276 | 2.35E+04 | 14.331 | 4.02E+04 |
| 1-octadecene | 1.75E+04 | 3.77E+04 | 18.44 | 3.23E+04 | 18.51 | 4.33E+04 |
| n-eicosane | 19.893 | 4.30E+04 | 19.924 | 5.32E+04 | 19.998 | 4.77E+04 |
| Palmolive Pure & Clear | Mass of oiled feathers (g) | 2.04 | Mass of oiled feathers (g) | 2.04 | Mass of oiled feathers (g) | 2.03 |
| o-xylene | 6.445 | 3.71E+04 | 6.461 | 2.13E+04 | 6.511 | 4.07E+04 |
| mesitylene | 7.564 | 5.04E+04 | 7.581 | 3.14E+04 | 7.634 | 5.54E+04 |
| n-butylcyclohexane | 8.767 | 3.81E+04 | 8.784 | 2.39E+04 | 8.84 | 3.96E+04 |
| n-butylbenzene | 8.985 | 3.23E+04 | 9.003 | 2.10E+04 | 9.058 | 3.53E+04 |
| tetralin | 10.782 | 5.88E+04 | 10.801 | 3.99E+04 | 10.862 | 6.49E+04 |
| naphthalene | 11.137 | 5.94E+04 | 11.157 | 4.08E+04 | 11.218 | 6.60E+04 |
| n-dodecane | 11.399 | 5.26E+04 | 11.417 | 3.58E+04 | 11.476 | 5.72E+04 |
| tetradecane | 14.26 | 5.88E+04 | 14.28 | 4.13E+04 | 14.337 | 6.41E+04 |
| 1-octadecene | 18.407 | 3.46E+04 | 18.441 | 3.11E+04 | 18.515 | 4.15E+04 |
| n-eicosane | 19.896 | 1.76E+04 | 19.928 | 1.92E+04 | 20.007 | 2.39E+04 |
| Bright Green | Mass of oiled feathers (g) | 2.01 | Mass of oiled feathers (g) | 2.08 | Mass of oiled feathers (g) | 2.01 |
| o-xylene | 6.439 | 1.66E+04 | 6.465 | 2.01E+04 | 6.513 | 1.39E+04 |
| mesitylene | 7.557 | 2.44E+04 | 7.584 | 2.97E+04 | 7.634 | 1.87E+04 |
| n-butylcyclohexane | 8.76 | 1.82E+04 | 8.788 | 2.23E+04 | 8.84 | 1.34E+04 |
| n-butylbenzene | 8.98 | 1.64E+04 | 9.006 | 1.99E+04 | 9.059 | 1.20E+04 |
| tetralin | 10.778 | 3.07E+04 | 10.805 | 3.75E+04 | 10.86 | 2.20E+04 |
| naphthalene | 11.131 | 3.14E+04 | 11.159 | 3.88E+04 | 11.216 | 2.25E+04 |
| n-dodecane | 11.394 | 2.67E+04 | 11.423 | 3.40E+04 | 11.476 | 1.91E+04 |
| tetradecane | 14.258 | 3.01E+04 | 14.289 | 3.91E+04 | 14.336 | 2.18E+04 |
| 1-octadecene | 18.413 | 3.29E+04 | 18.453 | 3.55E+04 | 18.521 | 3.14E+04 |
| n-eicosane | 19.899 | 1.48E+04 | 19.939 | 1.42E+04 | 20.011 | 1.59E+04 |
| Dawn Platinum Powerwash | Mass of oiled feathers (g) | 2.02 | Mass of oiled feathers (g) | 1.57 |  |  |
| o-xylene | 6.455 | 4.97E+04 | 6.461 | 3.09E+04 |  |  |
| mesitylene | 7.576 | 6.81E+04 | 7.58 | 4.86E+04 |  |  |
| n-butylcyclohexane | 8.78 | 5.21E+04 | 8.783 | 3.70E+04 |  |  |
| n-butylbenzene | 8.998 | 4.39E+04 | 9 | 3.24E+04 |  |  |
| tetralin | 10.801 | 8.01E+04 | 10.798 | 6.07E+04 |  |  |
| naphthalene | 11.156 | 8.04E+04 | 11.153 | 6.10E+04 |  |  |
| n-dodecane | 11.417 | 7.36E+04 | 11.413 | 5.49E+04 |  |  |
| Homesolv by CitraSolv | Mass of oiled feathers (g) | 2.05 | Mass of oiled feathers (g) | 2.00 | Mass of oiled feathers (g) | 2.04 |
| o-xylene | 6.466 | 6.21E+04 | 6.444 | 8.18E+04 | 6.522 | 7.61E+04 |
| mesitylene | 7.587 | 8.57E+04 | 7.565 | 1.17E+05 | 7.646 | 1.09E+05 |
| n-butylcyclohexane | 8.792 | 6.55E+04 | 8.768 | 8.89E+04 | 8.854 | 8.46E+04 |
| n-butylbenzene | 9.009 | 5.49E+04 | 8.985 | 7.62E+04 | 9.071 | 7.12E+04 |
| tetralin | 10.812 | 9.93E+04 | 10.788 | 1.39E+05 | 10.88 | 1.30E+05 |
| naphthalene | 11.167 | 9.94E+04 | 11.143 | 1.40E+05 | 11.237 | 1.31E+05 |
| n-dodecane | 11.427 | 8.90E+04 | 11.403 | 1.25E+05 | 11.495 | 1.22E+05 |
| tetradecane | 14.289 | 9.85E+04 | 14.263 | 1.39E+05 | 14.355 | 1.36E+05 |
| 1-octadecene | 18.439 | 3.32E+04 | 18.403 | 4.01E+04 | 18.53 | 3.97E+04 |
| n-eicosane | 19.932 | 2.67E+04 | 19.895 | 3.23E+04 | 20.032 | 2.56E+04 |
| Charlie's Soap | Mass of oiled feathers (g) | 2.01 | Mass of oiled feathers (g) | 2.02 | Mass of oiled feathers (g) | 2.01 |
| o-xylene | 6.467 | 8.62E+04 | 6.439 | 1.54E+05 | 6.527 | 1.44E+05 |
| mesitylene | 7.59 | 1.34E+05 | 7.563 | 2.14E+05 | 7.656 | 2.30E+05 |
| n-butylcyclohexane | 8.795 | 1.06E+05 | 8.765 | 1.59E+05 | 8.864 | 1.79E+05 |
| n-butylbenzene | 9.012 | 9.01E+04 | 8.982 | 1.38E+05 | 9.082 | 1.61E+05 |
| tetralin | 10.817 | 1.68E+05 | 10.787 | 2.51E+05 | 10.899 | 3.10E+05 |
| naphthalene | 11.175 | 1.70E+05 | 11.146 | 2.56E+05 | 11.258 | 3.20E+05 |
| n-dodecane | 11.433 | 1.57E+05 | 11.404 | 2.25E+05 | 11.521 | 2.81E+05 |
| tetradecane | 14.294 | 1.76E+05 | 14.264 | 2.47E+05 | 14.371 | 3.10E+05 |
| 1-octadecene | 18.44 | 3.70E+04 | 18.395 | 4.51E+04 | 18.531 | 5.04E+04 |
| n-eicosane | 19.931 | 4.11E+04 | 19.889 | 7.21E+04 | 20.031 | 8.62E+04 |
| Fairy Liquid | Mass of oiled feathers (g) | 2.06 | Mass of oiled feathers (g) | 2.06 | Mass of oiled feathers (g) | 2.04 |
| o-xylene | 6.465 | 1.44E+04 | 6.435 | 1.62E+04 | 6.524 | 1.59E+04 |
| mesitylene | 7.584 | 2.02E+04 | 7.552 | 2.38E+04 | 7.647 | 2.20E+04 |
| n-butylcyclohexane | 8.787 | 1.55E+04 | 8.755 | 1.74E+04 | 8.857 | 1.47E+04 |
| n-butylbenzene | 9.006 | 1.32E+04 | 8.971 | 1.58E+04 | 9.075 | 1.42E+04 |
| tetralin | 10.805 | 2.46E+04 | 10.768 | 3.02E+04 | 10.879 | 2.62E+04 |
| naphthalene | 11.159 | 2.58E+04 | 11.121 | 3.19E+04 | 11.235 | 2.73E+04 |
| n-dodecane | 11.421 | 2.29E+04 | 11.385 | 2.75E+04 | 11.495 | 2.30E+04 |
| tetradecane | 14.282 | 2.68E+04 | 14.248 | 3.12E+04 | 14.355 | 2.71E+04 |
| 1-octadecene | 18.441 | 3.21E+04 | 18.417 | 4.47E+04 | 18.544 | 4.36E+04 |
| n-eicosane | 19.926 | 3.08E+04 | 19.904 | 5.44E+04 | 20.036 | 4.63E+04 |
| Dr. Bronner's | Mass of oiled feathers (g) | 2.07 | Mass of oiled feathers (g) | 2.05 | Mass of oiled feathers (g) | 2.00 |
| o-xylene | 6.469 | 2.12E+05 | 6.465 | 1.91E+05 | 6.533 | 1.63E+05 |
| mesitylene | 7.597 | 2.77E+05 | 7.594 | 2.55E+05 | 7.665 | 2.53E+05 |
| n-butylcyclohexane | 8.799 | 2.05E+05 | 8.8 | 1.94E+05 | 8.871 | 1.96E+05 |
| n-butylbenzene | 9.017 | 1.75E+05 | 9.016 | 1.62E+05 | 9.089 | 1.70E+05 |
| tetralin | 10.828 | 3.15E+05 | 10.83 | 2.92E+05 | 10.905 | 3.16E+05 |
| naphthalene | 11.189 | 3.20E+05 | 11.188 | 2.93E+05 | 11.266 | 3.22E+05 |
| n-dodecane | 11.443 | 2.76E+05 | 11.445 | 2.67E+05 | 11.519 | 2.91E+05 |
| tetradecane | 14.304 | 3.03E+05 | 14.308 | 2.93E+05 | 14.379 | 3.22E+05 |
| 1-octadecene | 18.435 | 3.84E+04 | 18.452 | 3.84E+04 | 18.537 | 4.41E+04 |
| n-eicosane | 19.931 | 6.77E+04 | 19.951 | 7.52E+04 | 20.039 | 8.28E+04 |
| Palmolive Ultra | Mass of oiled feathers (g) | 2.00 | Mass of oiled feathers (g) | 2.01 | Mass of oiled feathers (g) | 2.01 |
| o-xylene | 6.459 | 1.07E+04 | 6.479 | 1.17E+04 | 6.525 | 1.51E+04 |
| mesitylene | 7.576 | 1.50E+04 | 7.599 | 1.69E+04 | 7.647 | 2.18E+04 |
| n-butylcyclohexane | 8.78 | 1.09E+04 | 8.805 | 1.19E+04 | 8.855 | 1.55E+04 |
| n-butylbenzene | 8.997 | 9812.572 | 9.023 | 1.12E+04 | 9.073 | 1.45E+04 |
| tetralin | 10.795 | 1.81E+04 | 10.824 | 2.10E+04 | 10.876 | 2.70E+04 |
| naphthalene | 11.15 | 1.86E+04 | 11.18 | 2.14E+04 | 11.231 | 2.74E+04 |
| n-dodecane | 11.411 | 1.52E+04 | 11.441 | 1.80E+04 | 11.491 | 2.32E+04 |
| tetradecane | 14.274 | 1.66E+04 | 14.304 | 2.14E+04 | 14.352 | 2.74E+04 |
| 1-octadecene | 18.434 | 3.65E+04 | 18.477 | 3.52E+04 | 18.537 | 4.43E+04 |
| n-eicosane | 19.918 | 4.29E+04 | 19.964 | 4.40E+04 | 20.026 | 4.64E+04 |
| Ultra Joy | Mass of oiled feathers (g) | 2.01 | Mass of oiled feathers (g) | 2.01 | Mass of oiled feathers (g) | 2.00 |
| o-xylene | 6.462 | 5.69E+04 | 6.493 | 5.29E+04 | 6.522 | 8.34E+04 |
| mesitylene | 7.583 | 8.11E+04 | 7.616 | 7.37E+04 | 7.646 | 1.24E+05 |
| n-butylcyclohexane | 8.786 | 6.40E+04 | 8.821 | 5.55E+04 | 8.851 | 9.78E+04 |
| n-butylbenzene | 9.004 | 5.30E+04 | 9.039 | 4.77E+04 | 9.069 | 8.23E+04 |
| tetralin | 10.806 | 9.71E+04 | 10.843 | 8.74E+04 | 10.875 | 1.53E+05 |
| naphthalene | 11.161 | 9.75E+04 | 11.198 | 8.84E+04 | 11.232 | 1.54E+05 |
| n-dodecane | 11.422 | 9.16E+04 | 11.458 | 8.03E+04 | 11.488 | 1.44E+05 |
| tetradecane | 14.283 | 1.03E+05 | 14.319 | 8.97E+04 | 14.343 | 1.60E+05 |
| 1-octadecene | 18.435 | 3.59E+04 | 18.487 | 3.69E+04 | 18.503 | 4.07E+04 |
| n-eicosane | 19.927 | 2.38E+04 | 19.982 | 2.31E+04 | 19.996 | 4.43E+04 |
| Day 2 |  |  |  |  |  |  |
| Control | Mass of oiled feathers (g) | 1.62 |  |  |  |  |
| o-xylene | 6.472 | 1.43E+05 |  |  |  |  |
| mesitylene | 7.598 | 2.26E+05 |  |  |  |  |
| n-butylcyclohexane | 8.803 | 1.81E+05 |  |  |  |  |
| n-butylbenzene | 9.018 | 1.53E+05 |  |  |  |  |
| tetralin | 10.829 | 2.87E+05 |  |  |  |  |
| naphthalene | 11.186 | 2.90E+05 |  |  |  |  |
| n-dodecane | 11.444 | 2.69E+05 |  |  |  |  |
| tetradecane | 14.304 | 2.98E+05 |  |  |  |  |
| 1-octadecene | 18.442 | 4.22E+04 |  |  |  |  |
| n-eicosane | 19.942 | 6.85E+04 |  |  |  |  |
| Dawn Platinum Powerwash | Mass of oiled feathers (g) | 2.09 | Mass of oiled feathers (g) | 2.00 | Mass of oiled feathers (g) | 2.04 |
|  | retention time (min) | area count | retention time (min) | area count | retention time (min) | area count |
| o-xylene | 6.676 | 4.87E+04 | 6.536 | 5.17E+04 | 6.491 | 7.00E+04 |
| mesitylene | 7.804 | 7.05E+04 | 7.659 | 7.52E+04 | 7.614 | 9.66E+04 |
| n-butylcyclohexane | 9.016 | 5.40E+04 | 8.865 | 5.66E+04 | 8.82 | 7.20E+04 |
| n-butylbenzene | 9.234 | 4.61E+04 | 9.083 | 4.97E+04 | 9.038 | 6.23E+04 |
| tetralin | 11.045 | 8.50E+04 | 10.888 | 9.24E+04 | 10.842 | 1.14E+05 |
| naphthalene | 11.402 | 8.56E+04 | 11.244 | 9.33E+04 | 11.197 | 1.15E+05 |
| n-dodecane | 11.653 | 7.66E+04 | 11.501 | 8.37E+04 | 11.456 | 1.05E+05 |
| tetradecane | 14.501 | 8.41E+04 | 14.358 | 9.41E+04 | 14.316 | 1.18E+05 |
| 1-octadecene | 18.71 | 3.65E+04 | 18.532 | 4.29E+04 | 18.477 | 5.33E+04 |
| n-eicosane | 20.207 | 4.12E+04 | 20.024 | 3.51E+04 | 19.971 | 4.68E+04 |
| Fairy Liquid | Mass of oiled feathers (g) | 2.03 | Mass of oiled feathers (g) | 2.03 | Mass of oiled feathers (g) | 2.03 |
| o-xylene | 6.612 | 2.02E+04 | 6.513 | 1.27E+04 | 6.484 | 1.46E+04 |
| mesitylene | 7.739 | 2.68E+04 | 7.634 | 1.88E+04 | 7.602 | 2.09E+04 |
| n-butylcyclohexane | 8.95 | 1.97E+04 | 8.842 | 1.38E+04 | 8.807 | 1.50E+04 |
| n-butylbenzene | 9.168 | 1.71E+04 | 9.061 | 1.26E+04 | 9.025 | 1.37E+04 |
| tetralin | 10.975 | 3.11E+04 | 10.863 | 2.38E+04 | 10.823 | 2.56E+04 |
| naphthalene | 11.333 | 3.17E+04 | 11.218 | 2.46E+04 | 11.178 | 2.63E+04 |
| n-dodecane | 11.586 | 2.72E+04 | 11.477 | 2.15E+04 | 11.439 | 2.26E+04 |
| tetradecane | 14.44 | 3.07E+04 | 14.336 | 7.68E+03 | 14.299 | 2.62E+04 |
| 1-octadecene | 18.64 | 3.46E+04 | 18.517 | 3.56E+04 | 18.467 | 3.69E+04 |
| n-eicosane | 20.132 | 2.27E+04 | 20.004 | 2.13E+04 | 19.954 | 2.50E+04 |
| Palmolive Ultra | Mass of oiled feathers (g) | 2.09 | Mass of oiled feathers (g) | 2.01 | Mass of oiled feathers (g) | 2.09 |
| o-xylene | 6.58 | 9.34E+03 | 6.506 | 1.17E+04 | 6.475 | 1.46E+04 |
| mesitylene | 7.702 | 1.47E+04 | 7.626 | 1.77E+04 | 7.594 | 2.34E+04 |
| n-butylcyclohexane | 8.911 | 1.12E+04 | 8.831 | 1.28E+04 | 8.798 | 1.51E+04 |
| n-butylbenzene | 9.13 | 1.01E+04 | 9.049 | 1.20E+04 | 9.015 | 1.64E+04 |
| tetralin | 10.933 | 1.93E+04 | 10.849 | 2.29E+04 | 10.812 | 3.27E+04 |
| naphthalene | 11.288 | 1.97E+04 | 11.204 | 2.35E+04 | 11.168 | 3.42E+04 |
| n-dodecane | 11.545 | 1.70E+04 | 11.464 | 2.03E+04 | 11.429 | 2.84E+04 |
| tetradecane | 14.401 | 1.95E+04 | 14.322 | 2.39E+04 | 14.289 | 3.40E+04 |
| 1-octadecene | 18.6 | 3.60E+04 | 18.496 | 4.16E+04 | 18.455 | 4.30E+04 |
| n-eicosane | 20.091 | 1.09E+04 | 19.987 | 1.07E+04 | 19.945 | 1.24E+04 |
| Seventh Generation | Mass of oiled feathers (g) | 2.04 | Mass of oiled feathers (g) | 2.03 | Mass of oiled feathers (g) | 2.05 |
| o-xylene | 6.76 | 8.42E+04 | 6.557 | 8.26E+04 | 6.5 | 7.76E+04 |
| mesitylene | 7.893 | 1.26E+05 | 7.684 | 1.21E+05 | 7.624 | 1.28E+05 |
| n-butylcyclohexane | 9.109 | 9.78E+04 | 8.891 | 9.08E+04 | 8.831 | 1.00E+05 |
| n-butylbenzene | 9.327 | 8.48E+04 | 9.108 | 8.00E+04 | 9.048 | 8.85E+04 |
| tetralin | 11.148 | 1.58E+05 | 10.918 | 1.49E+05 | 10.856 | 1.67E+05 |
| naphthalene | 11.508 | 1.60E+05 | 11.275 | 1.50E+05 | 11.212 | 1.69E+05 |
| n-dodecane | 11.752 | 1.45E+05 | 11.529 | 1.35E+05 | 11.47 | 1.52E+05 |
| tetradecane | 14.597 | 1.60E+05 | 14.386 | 1.49E+05 | 14.33 | 1.68E+05 |
| 1-octadecene | 18.818 | 3.69E+04 | 18.563 | 4.09E+04 | 18.489 | 4.11E+04 |
| n-eicosane | 20.317 | 6.20E+04 | 20.064 | 3.42E+04 | 19.986 | 3.68E+04 |